Material Safety Data Sheet

VORTEXCLAY

# Section 1: Chemical Product  And Company Identification

MSDS Name: Bentonite

Commercial Names: VORTEXCLAY SD-1, VORTECLAY SD-2; VORTEXCLAY 1000; VORTEXCLAY 38 and 34

Company Identification:

VORTEX ARGENTINA S.A

RIGLOS Nº 855 CAPITAL FEDERAL CP 1424 ARGENTINA

FOR INFORMATION CALL 0054-11-49250044

FAX 0054-11-49242948

### Section2-Composition, Information On Ingredients

| CAS # | CHEMICAL NAME | PERCENT | EINECS/ELINCS |
| --- | --- | --- | --- |
| 1302-78-9 | BENTONITE (ORGANIC CLAY) | 100 | 215-108-5 |

Hazard Symbols: None  listed.

Risk Phrases: None listed.

## Section 3 – Hazards Identification

#### EMERGENCY OVERVIEW

Appearance: Cream solid. Caution! May cause eye and skin irritation. May cause respiratory and digestive tract irritation. May cause cancer based on animal studies. The toxicological properties of this material have not been fully investigated.

Target Organs: None.

Potential Health Effects

Eye: May cause eye irritation.

Skin: May cause skin irritation. Chronic inhalation may cause lung changes, chest pain, breath shortness, and bronichitis.

Ingestion: Ingestion of large amounts may cause gastrointestinal irritation. The toxicological properties of this substance have not been fully investigated.

Inhalation: May cause respiratory tract irritation. The toxicological properties of this substance have not been fully investigated.

Chronic: May cause cancer according to animal studies. Chronic inhalation may

cause lung changes, chest pain, breath shortness, and bronichitis.

# Section 4 – First Aid Measures

Eyes: Flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower eyelids. Get medical aid.

Skin: Flush skin with plenty of soap and water for at least 15 minutes while removing contaminated clothing and shoes. Get medical aid if Irritation develops or persists.

Ingestion: If victim is conscious and alert, give 2-4 cupfuls of milk or water. Never give anything by mouth to an unconscious person. Get medical aid.

Inhalation: Remove from exposure to fresh air immediately. Get medical aid if cough or other symptoms appear.

Notes to Physician: Treat symptomatically and supportively.

Section 5 –Fire Fighting Mesures

General Information: As In any fire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent), and full protective gear.     Extinguishing Media: For small fires, use water spray, dry chemical, carbon dioxide or chemical foam.

Flash Point: Not applicable.

Autoignition Temperature: Not applicable.

Explosion Limits, Lower: Not available.

Upper: Not available.

NFPA Rating: Not published.

# Section 6 – Accidental Release Measure

General Information:  Use   proper   personal  protective  equipment  as  indicated  in  Section 8.

Spills/Leaks: Vacuum or sweep up material and place into a suitable disposal container. Avoid generating dusty conditions.

# Section 7 – Handling And Storage

Handling: Use with adequate ventilation. Avoid contact with skin and eyes. Avoid ingestion and inhalation.

Storage: Store in a cool, dry place. Store in a tightly closed container.

## Section 8 -  Exposure Controls Personal Protection

Engineering Controls: Use adequate ventilation to keep airborne concentrations low.

Exposure Limits

| Chemical Name | ACGIH | NIOSH | OSHA- Final PELs |
| --- | --- | --- | --- |
| Bentonite | None Listed | None Listed | None Listed |

OSHA Vacated PELs: BENTONITE: No OSHA Vacated PELs are listed for this chemical.

Personal Protective Equipment

Eyes: Wear appropriate protective eyeglasses or chemical safety goggles as described by OSHA's eye and face protection regulations in 29 CFR 1910.133 or European Standard EN166.

Skin: Wear appropriate gloves to prevent skin exposure.

Clothing: Wear appropriate protective clothing to minimize contact with skin.

Respirators: Follow the OSHA respirator regulations found in 29CFR 1910.134 or European Standard EN 149. Always use a NIOSH or European Standard EN 149 approved respirator when necessary.

Section 9 – Physical And Chemical Properties

Physical State: Solid

Appearance: Cream

Odor: odorless

pH: Not available.

Vapor Pressure: Not available.

Vapor Density: Not available.

Evaporation Rate: Not applicable.

Viscosity: Not available.

Boiling Point: Not available.

Freezing/Melting Point: Not available.

Decomposition Temperature: Not available.

Solubility: Insoluble in water.

Specific Gravity/Density. Not available.

Molecular Formula-Not applicable.

Molecular Weight. Not available.

# Section 10 – Stability And Reactivity

Chemical Stability: Stable under normal temperatures and pressures. Conditions to Avoid: Moisture,

Incompatibilities with Other Materials: None reported.

Hazardous Decomposition Products: Irritating and toxic fumes and gases. Hazardous Polymerization: Has not been reported.

# Section 11 – Toxicological Information

RTECS#:

CAS# 1302-78-9: CT9450000

LD50/LC50: Not available

Carcinogenicity.

CAS# 1302-78-9; Not listed by ACGIH, IARC, NIOSH, NTP, or OSHA.

Epidemiology: Experimental tumorigenic data has been reported.

Teratogenicity: No data available.

Reproductive Effects: No data available.

Neurotoxiclty: No data available.

Mutagenicity: No data available.

Other Studies: No data available.

# Section 12 – Ecological Informations

No information available.

## Section 13 – Disposal Considerations

Chemical waste  generators  must  determine  whether  a  discarded  chemical    is

classified    as   a  hazardous  waste.     US EPA  guidelines   for   the   classification

determination are listed in 40 CFR Parts 261.3. Additionally, waste generators must

consult state and local hazardous waste regulations to ensure complete and

accurate classification.

RCRA P-Series: None listed.

RCRA U-Series: None listed

## Section 14 – Transporte Information

None listed

## Section 15 – Regulatory Information

TSCA

CASE 1302-78-9 Is listed on the TSCA inventory.

Health & Safety Reporting List

None of the chemicals are on the Health & Safety Reporting List.

Clean Air Act:

This material does not contain any hazardous air pollutants, This material does not contain any Class 1 Ozone depletors. This material does not contain any Class 2 Ozone depletors.

Clean Water Act:

None of the chemicals In this product are listed as Hazardous Substances under the CWA. None of the chemicals in this product are listed as Priority Pollutants under the CWA. None of the chemicals in this product are listed as Toxic Pollutants under the CWA.

OSHA

None of the chemicals in this product are considered highly hazardous by OSHA.

# Section 16 – Additional  Information

The information above Is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitability of the Information for their particular purposes. In no event shall Fisher be liable for any claims, losses, or damages of any third party or for lost profits or any special, indirect, incidental, consequential or exemplary damages, howsoever arising, even if Fisher has been advised of the possibility of such damages.